### NAME 01 : NIRMAAN
NAME 02: The Breathing Office

Year: 2025
Location: NA
Type: Competition 
Status: NA 
Photographers: NA

Located within a warm and humid climatic context, NIRMAAN is conceived as a building that works with climate rather than against it. The design prioritises passive strategies, landscape integration, and material intelligence to create comfortable, naturally ventilated workspaces with minimal reliance on mechanical systems.

A reduced built-to-ground ratio allows the landscape to take precedence, enabling the creation of a dense Miyawaki forest within the site. This green mass extends into and over the building, forming a self-sustaining microclimate that cools the surroundings and enhances thermal comfort. The built form is carefully positioned, with service blocks along the western edge acting as a thermal buffer against the harsh afternoon sun. All workspace bays are oriented along the north–south axis, maximising soft daylight while promoting effective cross-ventilation throughout the day.

At the heart of the building, courtyards function as its lungs—inhaling and exhaling air while anchoring social interaction. These spaces draw from local cultural traditions, offering shaded, communal environments that balance work and pause. Above, staggered terraces transform the roofscape into layered green zones, while deep overhangs below provide shade and reduce heat gain. Automated louvers respond to user needs, regulating daylight and glare while maintaining visual comfort.

The roof system further strengthens the passive strategy. Jack-arch roofs with ventilated cavities allow hot air to escape naturally, enhancing stack ventilation and reducing dependence on artificial cooling. Material choices reinforce this approach. Earth walls increase thermal mass, moderating indoor temperatures while grounding the building in its context. The use of local, eco-friendly materials reduces embodied energy and strengthens the project’s relationship with place.

Nature is not treated as an addition but as an integral layer of the architecture. The raised built form enables both horizontal and vertical continuity of green spaces, allowing landscape to flow seamlessly through courtyards, terraces, and workspaces. This continuous interface between indoors and outdoors improves air quality, visual comfort, and the overall microclimate experienced within the building.

Performance studies—focusing on daylight, ventilation, and thermal comfort—validate the effectiveness of the design. The results demonstrate that carefully orchestrated passive strategies can deliver energy-efficient, healthy, and well-lit work environments, reaffirming the potential of climate-responsive architecture as a sustainable and humane approach to building design.